Authors Response (JCAS-D-24-00511)

A Negative Specified-Duration Shaper to Reduce Residual Vibration within Arbitrarily Specified Time less than Half Period by Brian Byunghyun Kang

The author is grateful for the Editor’s and the Reviewers’ constructive comments regarding the manuscript. I have done my best to answer all the questions raised by the Reviewers and have revised the manuscript as per his/her advices. The manuscript has been carefully reviewed and the author tried to include all the comments and concerns as much as possible. I have quoted below the Reviewer’s comments in order, and put my answers after each comment. I sincerely hope that the revised manuscript will be a worthy candidate for publication.

* The main modifications are colored in red in the revised manuscript, shown after the Authors Response.

* All the numbers of the figures, equations, references and tables in this response letter were written according to the numbers in the revised manuscript.

Comments from the editors

SE Comments:

Several reviewers have raised critical issues regarding the contribution and clarity of the work, particularly in relation to well-established input shaping technology. Significant improvements are required for the paper to be considered for publication.

AE Comments:

A negative specified duration input shaping method was presented in this paper. Four experts were reviewed this paper and raised several critical concerns, especially on contribution and clarification. Please revise the paper carefully by considering the reviewers comment and provide point-by-point response to each comment in a separate file.

Thank you very much for the comments. First of all, I have modified the manuscript to fulfill all the comments from the reviewers. In this response letter, I have included responses and actions to the all comments.

Reviewer: 1

Comments to the Authors:

A negative specified-duration shaper within an arbitrarily specified time of less than half a period is proposed in this paper. A step by step procedure of the SD shaper design is well explained in detail and performance is validated in simulation and experiments. However, if we recall that the traditional input shaping method also can be designed regarding arbitrarily time specification and can have a negative amplitude input shaping, the novelty or advancement in comparison with the conventional input shaping seems not significant. The author is suggested to clarify the novelty and the contribution of the study in comparison with the well-established input shaping should be addressed. Moreover, writings and presentations should be significantly improved for the possible further review.

Comments:

1) Please explain why the less than half period is important for controller design? As author mentioned in the introduction, isn't it more beneficial with the shorter control time of less than 30% or 40% of period?

Response: We greatly thank the reviewer for the valuable comments. As mentioned in the manuscript, the proposed system uses two different tendons, an actuation tendon and a sensing tendon which are depicted in Fig. 3. The actuation tendon is used to actuate soft wearable hand robots and measure tension on the actuation tendon at the same time. To measure tension on the actuation tendon, a loadcell implemented along the actuation tendon at the actuator side. Thus, one end of actuation tendons should be installed at the pulley of the actuator and the other should be installed at the thimble of the glove. Meanwhile, in order to embed BoASensor in the system, sensing tendon is required. As details of the BoASensor can be found in the references, BoASensor requires separate tendon to measure bending angle of the sheaths. The working principle of BoASensor is by measuring the displacement change of the sensing tendon caused by accumulated bending angles along the sheath. Thus, the sensing tendon to measure bending angle of the sheaths should be placed only inside the sheaths. If one end of the sensing tendon is placed at the thimble of the glove, finger movements will affect BoASensor and make it hard to distinguish sheath bending angle from finger movements. Therefore, unlike the actuation tendons, one end of the sensing tendon is installed at the sensor module, which is placed at the actuation side end of the sheath. The other end of the sensing tendon installed at the glove side end of the sheath. To clarify, the modification in the manuscript has been made. Action: The modifications made in the new manuscript are listed below: III. Methods – A. Hardware Setup 2) BoASensor (Paragraph 1) “Thus, the sensing tendon should be placed only inside the sheaths. Therefore, unlike the actuation tendons, one end of the sensing tendon is installed at the BoASensor module, which is placed at the actuation side end of the sheath. The other end of the sensing tendon installed at the glove side end of the sheath.”

2) It is recommended, firstly, to present the requirements of SD shaper and the given condition (parameters) before explaining the designing procedure for easy reproduction of readers.

Response: We thank the reviewer pointing out the unclear expressions about the BT term calculation that can strengthen our paper. We modified the expression as follows: BT=∇xt(7)∙tiTρt As seen, the BT term is calculated by the gradient of the bending angle at the flexion wire given time multiplied by tiT, to the power ρt. Since the input data are discrete, the gradient can simply be calculated by the difference between the bending angle value at t and the value at t-1, which can be expressed as ∆xt(7)=Δxt(7)-Δxt-1(7). ρt is a trainable power term; we train this term as well as the other LSTM weights. After calculations, T-1 number of BT terms is obtained for each operation, from t=2 to T. Each of BT Term is multiplied by the hidden output at t, ht, and memory output at time t, ct. In order to minimize the misunderstanding of the notations, we modified terms mentioned in <III. Method – C. Fingertip Force Learning 1) Input and Output Data> to accord the notations of LSTM: time step t becomes lowercase and the input numbers become uppercase with parentheses. Action: The modifications have been made in the manuscript as follows: - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “In a LSTM window with the assigned time window value T, an input data xt at each time stept, where t= 1,2,...,T, is composed as follows: xt={xt1, xt2, xt3,xt4,xt5,xt6,xt7,xt8} … and yT is the fingertip force at time T. A sequence of the sensor output x(1:T)={xt=1,xt=2,…,x(t=T)}” - III. Methods – C. Fingertip Force Learning 2) BT-LSTM The equation (6) has been modified as follows. BT=∇xt(7)∙tiTρt - III. Methods – C. Fingertip Force Learning 2) BT-LSTM (Paragraph 2) “… Unlike the usual LSTM architecture, the BT term in BT-LSTM calculates the gradient of bending angle at the flexion wire given time, ∇xt(7), which can be obtained by Δxt(7)-Δxt-1(7). This term is then multiplied by tiT where ti represents the current time index in the time window.”

3) Symbols and equations need significant improvements and proofreading, e.g. what is D after equation (17)?

Response: The forget gate decides what information to lose and what information to keep (expressed as ft in the equation). This concept has been proposed to deal with the long-term dependency (or long-range dependencies) issues in Recurrent Neural Network (RNN). RNN models usually have long sequence that causes vanishing or exploding gradient, as mentioned in Ian Goodfellow et al*. This is the reason why the forget gate is needed. In order to clarify this, we modify the manuscript as follows. * I. Goodfellow, Y. Bengio, and A. Courville, “Deep learning.” MIT press, 2016. Action: We added a paragraph in the discussion section to highlight this point. - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “The forget gate decides what information to keep or discard by multiplying 1 or 0 using a sigmoid unit.”

4) In step (vii), in what case the solution of (7) does not exist? and, how the additional impulse function can solve this problem? Because the unknown parameters are increased, the redundant problem may happen in (10).

Response: The forget gate decides what information to lose and what information to keep (expressed as ft in the equation). This concept has been proposed to deal with the long-term dependency (or long-range dependencies) issues in Recurrent Neural Network (RNN). RNN models usually have long sequence that causes vanishing or exploding gradient, as mentioned in Ian Goodfellow et al*. This is the reason why the forget gate is needed. In order to clarify this, we modify the manuscript as follows. * I. Goodfellow, Y. Bengio, and A. Courville, “Deep learning.” MIT press, 2016. Action: We added a paragraph in the discussion section to highlight this point. - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “The forget gate decides what information to keep or discard by multiplying 1 or 0 using a sigmoid unit.”

5) it is missing to explain how did the author obtain A_4 in (10).

Response: The forget gate decides what information to lose and what information to keep (expressed as ft in the equation). This concept has been proposed to deal with the long-term dependency (or long-range dependencies) issues in Recurrent Neural Network (RNN). RNN models usually have long sequence that causes vanishing or exploding gradient, as mentioned in Ian Goodfellow et al*. This is the reason why the forget gate is needed. In order to clarify this, we modify the manuscript as follows. * I. Goodfellow, Y. Bengio, and A. Courville, “Deep learning.” MIT press, 2016. Action: We added a paragraph in the discussion section to highlight this point. - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “The forget gate decides what information to keep or discard by multiplying 1 or 0 using a sigmoid unit.”

6) Please provide a background of 5% insensitivity selection criteria and why it is calculated in step (v).

Response: The forget gate decides what information to lose and what information to keep (expressed as ft in the equation). This concept has been proposed to deal with the long-term dependency (or long-range dependencies) issues in Recurrent Neural Network (RNN). RNN models usually have long sequence that causes vanishing or exploding gradient, as mentioned in Ian Goodfellow et al*. This is the reason why the forget gate is needed. In order to clarify this, we modify the manuscript as follows. * I. Goodfellow, Y. Bengio, and A. Courville, “Deep learning.” MIT press, 2016. Action: We added a paragraph in the discussion section to highlight this point. - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “The forget gate decides what information to keep or discard by multiplying 1 or 0 using a sigmoid unit.”

7) It reads that the experiments were aimed at reducing the container's residual vibration. However, the explanation and results were shown for the beam vibration. It should be clarified and revised.

Response: The forget gate decides what information to lose and what information to keep (expressed as ft in the equation). This concept has been proposed to deal with the long-term dependency (or long-range dependencies) issues in Recurrent Neural Network (RNN). RNN models usually have long sequence that causes vanishing or exploding gradient, as mentioned in Ian Goodfellow et al*. This is the reason why the forget gate is needed. In order to clarify this, we modify the manuscript as follows. * I. Goodfellow, Y. Bengio, and A. Courville, “Deep learning.” MIT press, 2016. Action: We added a paragraph in the discussion section to highlight this point. - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “The forget gate decides what information to keep or discard by multiplying 1 or 0 using a sigmoid unit.”

8) I would suggest depicting the actual control input of experiments in Fig. 12 and Fig. 13 for strong validation of the proposed approach.

Response: The forget gate decides what information to lose and what information to keep (expressed as ft in the equation). This concept has been proposed to deal with the long-term dependency (or long-range dependencies) issues in Recurrent Neural Network (RNN). RNN models usually have long sequence that causes vanishing or exploding gradient, as mentioned in Ian Goodfellow et al*. This is the reason why the forget gate is needed. In order to clarify this, we modify the manuscript as follows. * I. Goodfellow, Y. Bengio, and A. Courville, “Deep learning.” MIT press, 2016. Action: We added a paragraph in the discussion section to highlight this point. - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “The forget gate decides what information to keep or discard by multiplying 1 or 0 using a sigmoid unit.”

9) Writings need significant improvements. For example, implies, does, does, and does can be revised to be more technically readable, the font size in equations and figures are not consistent; some are not clearly visible and some are too big.

Response: The forget gate decides what information to lose and what information to keep (expressed as ft in the equation). This concept has been proposed to deal with the long-term dependency (or long-range dependencies) issues in Recurrent Neural Network (RNN). RNN models usually have long sequence that causes vanishing or exploding gradient, as mentioned in Ian Goodfellow et al*. This is the reason why the forget gate is needed. In order to clarify this, we modify the manuscript as follows. * I. Goodfellow, Y. Bengio, and A. Courville, “Deep learning.” MIT press, 2016. Action: We added a paragraph in the discussion section to highlight this point. - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “The forget gate decides what information to keep or discard by multiplying 1 or 0 using a sigmoid unit.”

10) Several symbols are out of line in the Text and Title of the figures. Overall equation and symbols should be revised to be shown clearly.

Response: The forget gate decides what information to lose and what information to keep (expressed as ft in the equation). This concept has been proposed to deal with the long-term dependency (or long-range dependencies) issues in Recurrent Neural Network (RNN). RNN models usually have long sequence that causes vanishing or exploding gradient, as mentioned in Ian Goodfellow et al*. This is the reason why the forget gate is needed. In order to clarify this, we modify the manuscript as follows. * I. Goodfellow, Y. Bengio, and A. Courville, “Deep learning.” MIT press, 2016. Action: We added a paragraph in the discussion section to highlight this point. - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “The forget gate decides what information to keep or discard by multiplying 1 or 0 using a sigmoid unit.”

Reviewer: 2

Comments to the Authors:

This paper introduces a new type of input shaper designed for time periods that are less than half of a complete cycle. While the existing SD shaper can be used for cases where the shaping time exceeds half of the period with positive impulses, the author suggests a negative specified-duration shaper to expand the applicability of the input shaper to situations where the shaping time is less than half a period. The paper is well-structured, easy to understand, and makes a clear contribution. However, there are some issues that need to be addressed.

Comments:

1) In the introduction, the author mentions that 'However, these input shapers are all input shapers with positive impulses, and thus the shaping time of the SI shaper is longer than one period, and the shaping time of the SD shaper is longer than half period'. This might be challenging to grasp for readers not well-versed in input shaping. It would be helpful if the author included a graphical abstract illustrating the differences between positive and negative impulses in the time domain, as well as a comparison of shapers with shaping times shorter or longer than half a period. Additionally, a qualitative explanation accompanied by the graphical abstract to illustrate why the proposed method is effective would be beneficial for readers.

Response: We greatly thank the reviewer for the valuable comments. As mentioned in the manuscript, the proposed system uses two different tendons, an actuation tendon and a sensing tendon which are depicted in Fig. 3. The actuation tendon is used to actuate soft wearable hand robots and measure tension on the actuation tendon at the same time. To measure tension on the actuation tendon, a loadcell implemented along the actuation tendon at the actuator side. Thus, one end of actuation tendons should be installed at the pulley of the actuator and the other should be installed at the thimble of the glove. Action: The modifications made in the new manuscript are listed below: III. Methods – A. Hardware Setup 2) BoASensor (Paragraph 1) “Thus, the sensing tendon should be placed only inside the sheaths. Therefore, unlike the actuation tendons, one end of the sensing tendon is installed at the BoASensor module, which is placed at the actuation side end of the sheath. The other end of the sensing tendon installed at the glove side end of the sheath.”

2) What are the specific advantages of the proposed method compared to the existing method in terms of time period? For instance, the well-known trade-off relationship between residual vibration compensation (Fig. 7) and sensitivity (Fig. 8) is a characteristic in the field of input shaping. What novel contributions does the proposed negative specified-duration shaper offer? Does it result in a shorter shaping time and faster stabilization of the moving object?

Response: We greatly thank the reviewer for the valuable comments. As mentioned in the manuscript, the proposed system uses two different tendons, an actuation tendon and a sensing tendon which are depicted in Fig. 3. The actuation tendon is used to actuate soft wearable hand robots and measure tension on the actuation tendon at the same time. To measure tension on the actuation tendon, a loadcell implemented along the actuation tendon at the actuator side. Thus, one end of actuation tendons should be installed at the pulley of the actuator and the other should be installed at the thimble of the glove. Action: The modifications made in the new manuscript are listed below: III. Methods – A. Hardware Setup 2) BoASensor (Paragraph 1) “Thus, the sensing tendon should be placed only inside the sheaths. Therefore, unlike the actuation tendons, one end of the sensing tendon is installed at the BoASensor module, which is placed at the actuation side end of the sheath. The other end of the sensing tendon installed at the glove side end of the sheath.”

3) Why does the negative SD shaper with a shaping time of less than half a period consist of three impulses, as shown in Fig. 2?

Response: We greatly thank the reviewer for the valuable comments. As mentioned in the manuscript, the proposed system uses two different tendons, an actuation tendon and a sensing tendon which are depicted in Fig. 3. The actuation tendon is used to actuate soft wearable hand robots and measure tension on the actuation tendon at the same time. To measure tension on the actuation tendon, a loadcell implemented along the actuation tendon at the actuator side. Thus, one end of actuation tendons should be installed at the pulley of the actuator and the other should be installed at the thimble of the glove. Action: The modifications made in the new manuscript are listed below: III. Methods – A. Hardware Setup 2) BoASensor (Paragraph 1) “Thus, the sensing tendon should be placed only inside the sheaths. Therefore, unlike the actuation tendons, one end of the sensing tendon is installed at the BoASensor module, which is placed at the actuation side end of the sheath. The other end of the sensing tendon installed at the glove side end of the sheath.”

4) The text size in the figure needs to be aligned with the caption of each figure.

Response: We greatly thank the reviewer for the valuable comments. As mentioned in the manuscript, the proposed system uses two different tendons, an actuation tendon and a sensing tendon which are depicted in Fig. 3. The actuation tendon is used to actuate soft wearable hand robots and measure tension on the actuation tendon at the same time. To measure tension on the actuation tendon, a loadcell implemented along the actuation tendon at the actuator side. Thus, one end of actuation tendons should be installed at the pulley of the actuator and the other should be installed at the thimble of the glove. Action: The modifications made in the new manuscript are listed below: III. Methods – A. Hardware Setup 2) BoASensor (Paragraph 1) “Thus, the sensing tendon should be placed only inside the sheaths. Therefore, unlike the actuation tendons, one end of the sensing tendon is installed at the BoASensor module, which is placed at the actuation side end of the sheath. The other end of the sensing tendon installed at the glove side end of the sheath.”

Reviewer: 3

Comments to the Author

This manuscript presents a new specified-duration (SD) shaper for the input of 2nd- order underdamped/undamped systems, in order to achieve vibration suppression. A particular focus lies in utilizing the negative-signed impulse in the design, with which the specified time can be reduced less than half of the period.

The proposed approach seems promising and its validity is well-verified via both simulation and experiment. Nonetheless, it is hard to grasp the main contributions of this work, as detailed explanations behind the design of the input shaper are mostly omitted. The author is recommended to revise the manuscript considering the comments listed below:

Comments:

1) In the current manuscript, the author begins by providing the impulse vector directly, without illustrating how these impulse vectors are utilized in the input shaping. Adding a preliminary section (that includes the basics on the input shaper and positive impulse vectors) might be helpful for possible readers.

Response: We greatly thank the reviewer for the valuable comments. While the main focus of this paper is to propose a method to estimate fingertip forces, we agree that showing the possibility of force control in accordance with fingertip forces obtained from the proposed method is also important. To comply with the reviewer’s comment, we implemented a Proportional-Integral-Derivative (PID) controller for force control as a feasibility study. We examined Action: The modifications made in the new manuscript are listed below: - IV. Result and Analysis (Paragraph 1) - Modified To evaluate the performance of the models, Root Mean Squared Error (RMSE), Mean Absolute Error (MAE), and Mean Absolute Percentage Error (MAPE) are used.

2) While the simulation and experimental results showed that taking a negative impulse improves the performance of the input shaper in ideal scenarios, it is not easy for me to understand the rationale behind such an improvement. For a similar reason, it seems also necessary to provide a detailed explanation on the guideline, in order to clarify its working principle. (For instance, the author suggested to follow Eq. (7). while there is no details on where these equations are actually coming from.)

The author is strongly recommended to clarify the underlying idea for design in the main body of the manuscript, and also in the simulation part.

Response: We greatly thank the reviewer for the valuable comments. While the main focus of this paper is to propose a method to estimate fingertip forces, we agree that showing the possibility of force control in accordance with fingertip forces obtained from the proposed method is also important. To comply with the reviewer’s comment, we implemented a Proportional-Integral-Derivative (PID) controller for force control as a feasibility study. We examined Action: The modifications made in the new manuscript are listed below: - IV. Result and Analysis (Paragraph 1) - Modified To evaluate the performance of the models, Root Mean Squared Error (RMSE), Mean Absolute Error (MAE), and Mean Absolute Percentage Error (MAPE) are used.

3) It is not simple for me to capture why taking ts less than (1/2)Td is difficult.

Response: We greatly thank the reviewer for the valuable comments. While the main focus of this paper is to propose a method to estimate fingertip forces, we agree that showing the possibility of force control in accordance with fingertip forces obtained from the proposed method is also important. To comply with the reviewer’s comment, we implemented a Proportional-Integral-Derivative (PID) controller for force control as a feasibility study. We examined Action: The modifications made in the new manuscript are listed below: - IV. Result and Analysis (Paragraph 1) - Modified To evaluate the performance of the models, Root Mean Squared Error (RMSE), Mean Absolute Error (MAE), and Mean Absolute Percentage Error (MAPE) are used.

4) While the author solved a set of (possibly nonlinear) equations with the trial-and- error approach, it can also be done automatically (not manually) by utilizing an optimization solver (some of which are user-friendly and thus can be easily applied to the problem of the author’s interest), including fmincon in MATLAB environment. From this aspect, it would be nice to consider use of an optimization tool in the framework. (I do not think optimization-based design is not necessarily presented for this work: thus, if such an improvement is not possible because of time limit, it is okay for me to leave it as a future work.)

Response: We greatly thank the reviewer for the valuable comments. While the main focus of this paper is to propose a method to estimate fingertip forces, we agree that showing the possibility of force control in accordance with fingertip forces obtained from the proposed method is also important. To comply with the reviewer’s comment, we implemented a Proportional-Integral-Derivative (PID) controller for force control as a feasibility study. We examined Action: The modifications made in the new manuscript are listed below: - IV. Result and Analysis (Paragraph 1) - Modified To evaluate the performance of the models, Root Mean Squared Error (RMSE), Mean Absolute Error (MAE), and Mean Absolute Percentage Error (MAPE) are used.

5) Why is the minimum value of Ai chosen as 0.01? Is there any specific reason for that

(e.g., this value has a physical meaning in some applications)?

Response: We greatly thank the reviewer for the valuable comments. While the main focus of this paper is to propose a method to estimate fingertip forces, we agree that showing the possibility of force control in accordance with fingertip forces obtained from the proposed method is also important. To comply with the reviewer’s comment, we implemented a Proportional-Integral-Derivative (PID) controller for force control as a feasibility study. We examined Action: The modifications made in the new manuscript are listed below: - IV. Result and Analysis (Paragraph 1) - Modified To evaluate the performance of the models, Root Mean Squared Error (RMSE), Mean Absolute Error (MAE), and Mean Absolute Percentage Error (MAPE) are used.

6) Many symbols used in the manuscript are not well-defined before their uses.

Response: We greatly thank the reviewer for the valuable comments. While the main focus of this paper is to propose a method to estimate fingertip forces, we agree that showing the possibility of force control in accordance with fingertip forces obtained from the proposed method is also important. To comply with the reviewer’s comment, we implemented a Proportional-Integral-Derivative (PID) controller for force control as a feasibility study. We examined Action: The modifications made in the new manuscript are listed below: - IV. Result and Analysis (Paragraph 1) - Modified To evaluate the performance of the models, Root Mean Squared Error (RMSE), Mean Absolute Error (MAE), and Mean Absolute Percentage Error (MAPE) are used.

7) In Eq. (23) (and also in the related equations), please rewrite the expression of [ti , Ai] as

tiAi= 01.71, 0.0425-1.2880, ⋯

instead of the current form (that seems to follow the rule of programming languages).

Response: We greatly thank the reviewer for the valuable comments. While the main focus of this paper is to propose a method to estimate fingertip forces, we agree that showing the possibility of force control in accordance with fingertip forces obtained from the proposed method is also important. To comply with the reviewer’s comment, we implemented a Proportional-Integral-Derivative (PID) controller for force control as a feasibility study. We examined Action: The modifications made in the new manuscript are listed below: - IV. Result and Analysis (Paragraph 1) - Modified To evaluate the performance of the models, Root Mean Squared Error (RMSE), Mean Absolute Error (MAE), and Mean Absolute Percentage Error (MAPE) are used.

8) In the title of Section 3, it seems nice to replace the term “analysis” with “verification”.

Response: We greatly thank the reviewer for the valuable comments. While the main focus of this paper is to propose a method to estimate fingertip forces, we agree that showing the possibility of force control in accordance with fingertip forces obtained from the proposed method is also important. To comply with the reviewer’s comment, we implemented a Proportional-Integral-Derivative (PID) controller for force control as a feasibility study. We examined Action: The modifications made in the new manuscript are listed below: - IV. Result and Analysis (Paragraph 1) - Modified To evaluate the performance of the models, Root Mean Squared Error (RMSE), Mean Absolute Error (MAE), and Mean Absolute Percentage Error (MAPE) are used.

Reviewer: 4

Comments to the Authors:

In this paper, a negative specified duration shaper with negative impulse has been proposed for reducing residual vibration within a short period. Under certain intuition, the result presents an algorithm with which it enables SD shaper with a short period of impulses.

Generally, the paper seems fairly carefully written. The parameters used in the design procedure seem well defined and derived, so that the procedure seems reproducible.

Comments:

1) Some possible concerns regarding the presentation would be that it might be hard to get the underlying intuition based on which the design procedure has been proposed and short period of impulse control is enabled. The paper includes simulation results as well as experimental validation so that the proposed method may seem convincing, but it could be commented that the paper may seem to lack theoretical reasoning.

Response: We greatly thank the reviewer for the valuable comments. The essential idea of the proposed method is to estimate fingertip forces in real-time without sensors on the glove. This paper also tries to verify the robustness of the proposed method by showing the performance under dynamic environments. BT-LSTM, the proposed deep learning algorithm, integrates dynamic information of motor encoders, wire tension, and sheath bending angles. To clarify the essential idea of the proposed method, we have modified the abstract. Furthermore, we modified the abstract to better address the issue we are trying to solve, which will help to understand the essential idea of the proposed method. Also, the abstract was modified to include quantitative evaluation results. After submission, we have conducted two further experiments to analyze performance of the proposed method. The first experiment was conducted to verify the repeatability of the method. Three different test datasets, which were collected on a different day after re-assembling the actuation system. Even with the new datasets, the proposed method showed best performance compared to the other methods. The second experiment was conducted to show the real-time applicability and force control. A PID controller was implemented and tested using 5-second step function. The detailed results of the experiments were included in the manuscript. Based on these results, the abstract was modified to present quantitative results. Action: The modifications made in the new manuscript are listed below: - Abstract - Modified “Soft wearable hand robots with tendon-sheath mechanisms are being actively developed to assist people with lost hand mobility. For these robots, accurately estimating fingertip forces leads to successful object grasping. An approach can utilize information from actuators assuming quasi-static environments. However, non-linearity and hysteresis with regards to the dynamic changes of the tendon-sheath mechanism hinder accurate fingertip force estimation. This paper proposes a learning-based method to estimate fingertip forces by integrating dynamic information of motor encoders, wire tension, and sheath bending angles. The model is modified from Long Short-Term Memory by incorporating a residual term that governs the dynamic changes in sheath bending angles. Using a tendon-driven soft wearable hand robot, the proposed model obtained RMSE less than 0.44N. It was further evaluated under criteria ranging from different object sizes, bending angle ranges, and forces. Finally, a repeatability test (0.46N in RMSE), real-time applicability (125 Hz), and force control (12.7% in MAPE) were performed to verify the feasibility of the proposed method.”