Authors Response (Robotics and Automation Letters)

Learning-Based Fingertip Force Estimation for Soft Wearable Hand Robot with Tendon-Sheath Mechanism by Kyu-Jin Cho, Sungho Jo, Brian Byunghyun Kang, Daekyum Kim, Hyungmin Choi, Useok Jeong, Kyu Bum Kim

The authors are grateful for the Editor’s and the Reviewers’ constructive comments regarding the manuscript. We have done our best to answer all the questions raised by the Reviewers and have revised the manuscript as per his/her advices. The paper has been carefully reviewed and the authors tried to include all the comments and concerns as much as possible. We have quoted below the Reviewer’s comments in order, and put our answers after each comment.

The authors thank the Reviewers for providing such constructive comments, which have definitely improved the quality of this paper. We have also prepared a response sheet wherein all the questions by the Reviewer have been addressed and the manuscript has been carefully revised. We sincerely hope that our revised manuscript will be a worthy candidate for publication.

* The main modifications are colored in red in the revised manuscript, shown after the Authors Response.

* All the numbers of the figures, equations, references and tables in this response letter were written according to the numbers in the revised manuscript.

Associate Editor

Comments to the Authors:

The core of this paper is to utilize a deep learning approach to estimate "finger tips forces".

Reviewer 3 raises a valid point regarding evaluation and how many data points that is used to assess the approach, currently a single dataset is used. As the contribution mainly lies in the combination of different known existing methods, the evaluation and discussion on the results are there for very important (for example, more discussion in Fig 7 would be helpful [reviewer 6]. As also pointed out "the authors should report relative errors in the force control alongside the already mentioned RMSE and MAE" is of importance.

Thank you very much for the insightful comments. First of all, we have modified the manuscript to fulfill all the comments from the reviewers. In this response letter, we have included responses and actions to the all comments. As reviewer 3 mentioned, we have performed additional experiments and included a section in the manuscript, <IV. Results and Analysis – F. Further Analysis 5: Repeatability>. In order to see repeatable performance of the proposed method, additional experiments were performed to gather three different test datasets. Each of the test datasets was collected 1) in accordance with the aforementioned data collection method, 2) on a different day, and 3) by re-assembling the actuation system. The results are tabulated in table with MAE, RMSE, and the Standard Deviation of the absolute error. The test datasets were compared with LSTM and LSTM+BoA. We believe that our main contribution of the study is to present the learning-based method using new deep learning algorithm (BT-LSTM) that can handle nonlinear behavior of robot systems with remote tendon-driven actuator system under dynamic environments. To clarify the contribution of the paper, we have modified our <Conclusion> section to <Discussion and Conclusion> section to further discuss our study. We have also added analysis on the results of the experiments and enhanced our figures in the <Results and Analysis> section. As reviewer 3 mentioned, for the comment related to force control, we have conducted a force control experiment and included in the manuscript, < IV. Results and Analysis – G. Further Analysis 6: Real-time Applicability and Force Control >. The experiment was performed using PID controller and relative errors have been presented as Mean Absolute Percentage Error. The experiment results were presented by referencing the reference [22].

It is furthermore also essential to compare the suggested approach with some base-line (preferably some recent work) this in order to give a better understanding of the benefits and also the drawbacks of the proposed system. It is essential that more care is spent on these issues.

We agreed that the previous manuscript needed more discussion on the study for a better understanding of the benefits and drawbacks of the proposed system. As already mentioned, we have modified our <Conclusion> section to <Discussion and Conclusion> section to include further discussion in the manuscript. We have carefully revised the reviewers’ comments and added current limitations of the proposed system and possible future approaches for the limitations. We also have included the applicability of the proposed method in the section for a better understanding of our paper.

The paper has a good structure, some parts needs further clarifications as pointed out by the reviewers (for example the BT term as pointed out by reviewer 1). The pictures are helpful but some needs further improvement for readability. Also as pointed out by reviewer 6 more explanations is needed w.r.t. the selected approach.

From the overall comments, we have noticed further clarifications on the algorithm explanation would be necessary. For this reason, we have modified the algorithm explanation section, especially related to BT term, to give better understandings. For the figures, we have modified most of our figures based on the multiple comments from the reviewers. To conclude, we have carefully revised all the comments from the associate editor and reviewers, and tried our best to improve our manuscript in the limited space. For further detailed modifications made in the manuscript, please check the following response letter.

Reviewer: 1

Comments to the Authors:

The studies on the soft wearable hand robot with tendon sheath mechanism are current topics nowadays. The authors basically worked on the correct fingertip force estimation using the available system based on a modified learning algorithm. They first showed the parameters affecting the correctness of measurement at fingers and then made the comparison among LSTM, LSTM+BoA and BT-LSTM.

Major concerns:

1) The thing that does not clear to me is why the system requires different types of tendons instead of one. Here, the actuation tendon can also be employed for the task of sensing tendon as well. Why not?

Response: We greatly thank the reviewer for the valuable comments. As mentioned in the manuscript, the proposed system uses two different tendons, an actuation tendon and a sensing tendon which are depicted in Fig. 3. The actuation tendon is used to actuate soft wearable hand robots and measure tension on the actuation tendon at the same time. To measure tension on the actuation tendon, a loadcell implemented along the actuation tendon at the actuator side. Thus, one end of actuation tendons should be installed at the pulley of the actuator and the other should be installed at the thimble of the glove. Meanwhile, in order to embed BoASensor in the system, sensing tendon is required. As details of the BoASensor can be found in the references, BoASensor requires separate tendon to measure bending angle of the sheaths. The working principle of BoASensor is by measuring the displacement change of the sensing tendon caused by accumulated bending angles along the sheath. Thus, the sensing tendon to measure bending angle of the sheaths should be placed only inside the sheaths. If one end of the sensing tendon is placed at the thimble of the glove, finger movements will affect BoASensor and make it hard to distinguish sheath bending angle from finger movements. Therefore, unlike the actuation tendons, one end of the sensing tendon is installed at the sensor module, which is placed at the actuation side end of the sheath. The other end of the sensing tendon installed at the glove side end of the sheath. To clarify, the modification in the manuscript has been made. Action: The modifications made in the new manuscript are listed below: III. Methods – A. Hardware Setup 2) BoASensor (Paragraph 1) “Thus, the sensing tendon should be placed only inside the sheaths. Therefore, unlike the actuation tendons, one end of the sensing tendon is installed at the BoASensor module, which is placed at the actuation side end of the sheath. The other end of the sensing tendon installed at the glove side end of the sheath.”

2) The calculation of the BT term was not clearly expressed and explained.

Response: We thank the reviewer pointing out the unclear expressions about the BT term calculation that can strengthen our paper. We modified the expression as follows: BT=∇xt(7)∙tiTρt As seen, the BT term is calculated by the gradient of the bending angle at the flexion wire given time multiplied by tiT, to the power ρt. Since the input data are discrete, the gradient can simply be calculated by the difference between the bending angle value at t and the value at t-1, which can be expressed as ∆xt(7)=Δxt(7)-Δxt-1(7). ρt is a trainable power term; we train this term as well as the other LSTM weights. After calculations, T-1 number of BT terms is obtained for each operation, from t=2 to T. Each of BT Term is multiplied by the hidden output at t, ht, and memory output at time t, ct. In order to minimize the misunderstanding of the notations, we modified terms mentioned in <III. Method – C. Fingertip Force Learning 1) Input and Output Data> to accord the notations of LSTM: time step t becomes lowercase and the input numbers become uppercase with parentheses. Action: The modifications have been made in the manuscript as follows: - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “In a LSTM window with the assigned time window value T, an input data xt at each time stept, where t= 1,2,...,T, is composed as follows: xt={xt1, xt2, xt3,xt4,xt5,xt6,xt7,xt8} … and yT is the fingertip force at time T. A sequence of the sensor output x(1:T)={xt=1,xt=2,…,x(t=T)}” - III. Methods – C. Fingertip Force Learning 2) BT-LSTM The equation (6) has been modified as follows. BT=∇xt(7)∙tiTρt - III. Methods – C. Fingertip Force Learning 2) BT-LSTM (Paragraph 2) “… Unlike the usual LSTM architecture, the BT term in BT-LSTM calculates the gradient of bending angle at the flexion wire given time, ∇xt(7), which can be obtained by Δxt(7)-Δxt-1(7). This term is then multiplied by tiT where ti represents the current time index in the time window.”

3) What is the forget gate?

Response: The forget gate decides what information to lose and what information to keep (expressed as ft in the equation). This concept has been proposed to deal with the long-term dependency (or long-range dependencies) issues in Recurrent Neural Network (RNN). RNN models usually have long sequence that causes vanishing or exploding gradient, as mentioned in Ian Goodfellow et al*. This is the reason why the forget gate is needed. In order to clarify this, we modify the manuscript as follows. * I. Goodfellow, Y. Bengio, and A. Courville, “Deep learning.” MIT press, 2016. Action: We added a paragraph in the discussion section to highlight this point. - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “The forget gate decides what information to keep or discard by multiplying 1 or 0 using a sigmoid unit.”

Minor comments:

1) There are some typos:

- ABSTRACT: However, directly placing placing sensors--Remove the redundancy

- Section IIIA:  ...were placed between a loacell--- loadcell

Response: We thank the reviewer for the feedbacks. All the changes proposed by the reviewer have been implemented in the manuscript. Action: We implemented all changes suggested by the reviewer.

2) For the black and white print of the paper, the blue and black curves in Fig 2 could not be distinguishable. Pls, change the colour or character of one of both.

Response: We thank the reviewer for the feedbacks. The change proposed by the reviewer has been implemented in the manuscript. Action: Fig. 2 was modified as the reviewer suggested. In addition, other figures were also modified.

3) You should mention Figure 3 in the problem definition, would be easier to understand the phenomenon.

Response: We thank the reviewer for the insightful comment. We agreed to the reviewer’s comment and modified the manuscript as suggested. Action: The modifications made in the manuscript are as follows. - II. Problem Description – A. Problem Definition “To clearly address the concept of non-linear and hysteresis behaviors in the soft wearable hand robot with TSM, relationships between actuation lengths, flexion wire tension, and fingertip forces are plotted in Fig. 2. Among these factors, this study focuses on estimating fingertip forces considering sheath angle variation and object size variation for a single user. For the system with TSM, considering the non-linear tendon behavior caused by the sheath angle variation (Fig. 3) is critical for estimating and controlling the system.”

Reviewer: 3

Comments to the Author

The paper discusses a new approach to estimate the output force of remote actuation systems based on Bowden cables and sensors placed within the remote actuation unit. Instead of using a forward model as mostly done in state-of-the-art systems the authors use deep learning methods to train the system.

To make the accuracy of the force control comparable to existing state-of-the-art systems, the authors should report relative errors in the force control alongside the already mentioned RMSE and MAE. Afterwards, the authors should include a comparison against alternative, not deep-learning based approaches (e.g. Hofmann2018) into the discussion to show if their approach is capable of controlling the output force more accurately.

Response: We greatly thank the reviewer for the valuable comments. While the main focus of this paper is to propose a method to estimate fingertip forces, we agree that showing the possibility of force control in accordance with fingertip forces obtained from the proposed method is also important. To comply with the reviewer’s comment, we implemented a Proportional-Integral-Derivative (PID) controller for force control as a feasibility study. We examined the force controller using a 5-second step function as input signals and used Mean Absolute Error (MAE) and Mean Absolute Percentage Error (MAPE) as evaluation metrics, similar to Hofmann 2018. The result, depicted in Fig. 8, shows that the MAE between the desired force and measured force was 0.72N, which is 12.7% in MAPE. The desired force has been reached in about 1.2 seconds while it takes about 1.4 seconds to return back to 0N. Some error seems to come from slow actuations of the actuators, as the MAE became 0.31N, which is 3.69% in MAPE after reaching the target force. Unlike the other studies that are mentioned in the introduction literature review, in our study, a user was equipped with the wearable glove and performed experiments. This introduces additional unexpected uncertainties that may come from non-linearity of the user hand and wearable robot. This in sum makes it harder to accurately estimate fingertip forces, and we believe that our method successfully addresses this issue. On the other hand, while it is difficult to directly compare our system with the existing method like Hofmann et al. due to the distant goal and different robot actuation systems*, compared to non-deep-learning-based models, our method has some limitations that need to be further developed when it comes to force control. First, the proposed deep learning model may sometimes be cumbersome in time to control the output forces (real-time applicability on portable devices will be explained in the next response). This is due to the fact that deep learning models require a lot of computations, and as a matter of fact, computing devices also need to be compact in order to be used for wearable devices. Based on the control result, it took about 1.2 seconds to reach the desired force while Hofmann et al. got less than 0.5 second. Although there can be other reasons to explain this (i.e., the lower actuation speed and the existence of an actual hand), the control speed can be one reason. Therefore, if force control is necessary in addition to force estimations, a better controller dealing with such issues need to be further developed. Second, compared to modeling methods, our method is data-dependent. This means that it requires a lot of data and time, which can be consuming to train the proposed network. This can be mitigated by combining with prior knowledge like non-learning-based models, which will be one of our future works. *: Their system has rack-and-pinion units on the both end points. Action: The modifications made in the new manuscript are listed below: - A new section has been added in Result and Analysis. F. Further Analysis 6: Real-time applicability and controllability - The section title “Conclusions” has been modified to “Discussion and Conclusion”. - Fig. 8 (Force control results using the step function) has been inserted. - IV. Result and Analysis (Paragraph 1) - Modified To evaluate the performance of the models, Root Mean Squared Error (RMSE), Mean Absolute Error (MAE), and Mean Absolute Percentage Error (MAPE) are used. - IV. Results and Analysis, G. Further Analysis 6: Real-time applicability and Force Control (Paragraph 2) -Added “To validate that the proposed system enables force control, a PID controller was implemented and tested under the laptop setting. The controller was tested using a 5-second step function as input signals, as shown in Fig. 8. MAE between the desired and measured force was 0.72N, which is 12.7% in MAPE. The desired force has been reached in about 1.2 seconds while it takes about 1.4 seconds to return back to the rest. Compared to Hofmann et al. [22], whose performance was 6.6% in MAPE for the step response, this error seems a bit higher although they used a different actuation system. It is assumed that the biggest error of the proposed method is due to the slower actuation system; the time taken to the force setpoint was more than twice as slow as Hofmann et al. It is notable that after reaching the desired force, MAE became 0.31N, which is 3.69% in MAPE.” - V. Discussion and Conclusion (Paragraph 1) - Modified “… The ability of force control using the proposed method is also examined.” - V. Discussion and Conclusion (Paragraph 3) - Added “Although the proposed method outperformed the other learning-based models in estimating fingertip forces, there still are some limitations and challenges that need to be further developed. First, the proposed method needs vast calculations during inferences compared to existing non-deep-learning-based methods. For force control, this study used the laptop as a computing device, which is small enough to fit in wheelchairs. However, since the computation speed of the embedded devices was not fast enough, this kind of task may not currently be manageable. Second, the proposed method is data-dependent. It requires a lot of data for training the network and takes long training time. To overcome this, integrating the proposed approach with a prior knowledge using existing methods, like [22], would be necessary.”

The method was used to predict and estimate output forces offline after acquiring the data. Nevertheless, for exoskeletons this method needs to run in real time on a microcontroller. The authors should show that their system is capable of estimating output forces in real time on a microcontroller to prove usability of the approach in activities of a daily living.

Response: We appreciate the reviewer’s comment regarding real-time applicability on a portable device. The target of the wearable glove as well as the proposed method is to assist Spinal Cord Injury (SCI) patients for their Activities of Daily Living (ADL). In this reason, the computing devices need to be compact such that they can fit in wheelchairs. We tested two different devices that satisfy this: a laptop (Asus ROX Zephyrus GX501, 15.6-inch, 2.2kg, NVIDIA GeForce GTX 1080 Graphics unit) and an embedded device (NVIDIA Jetson AGX Xavier, size: 105mm by 105mm, 280 g, 512-core Volta GPU). The running speed on the laptop was 125Hz and on the embedded device was 22.2Hz. We performed the control test mentioned above under the laptop setting. If the main purpose of force estimations is to perform real-time control, the speed of the NVIDIA embedded device may not be fast enough. The wearable device we used is for SCI patients, and thus users do not need to withstand the weights of the computing device and actuation systems (unless the device is big or heavy to fit in wheelchairs). Using the laptop, we were able to control the output forces although the speed may not as fast as non-deep-learning-based methods. However, for some other wearable devices like ones enhancing grasping forces* that require users to carry the whole devices, our method may currently not be applicable due to the speed issue. We expect that the development of imbedded devices with graphics units someday enables faster operations with lighter weights. Our manuscript has been modified accordingly. * Reference [7-10] in the manuscript Action: The modifications have been made as follows: - A new section has been added in Result and Analysis. F. Further Analysis 6: Real-time applicability and controllability - IV. Results and Analysis, G. Further Analysis 6: Real-time applicability and Force Control (Paragraph 1) -Added “This section is to examine the feasibility for an actual deployment of the proposed system. Two computing devices were tested for real-time applicability: a laptop (ROX Zephyrus GX501, Asus) and an embedded computing device (Jetson AGX Xavier, NVIDIA). On the laptop, the fingertip forces were estimated with 125Hz, and the embedded device gave 22.2Hz.” - V. Discussion and Conclusion (Paragraph 1) - Modified “… To validate the possibility of actual deployment, portable devices were tested.”

It seems that the evaluation is based on single datasets and measurements. To further improve it, the authors should repeat each measurement for at least three cases to showcase the repeatability of the procedure. Furthermore, instead of only reporting the mean errors over the timetrace, its standard deviation should be included into the results.

Response: We agree that the tests on more dataset can strengthen our argument. We performed additional experiments to gather three different cases. Each experiment performed on a different day and actuation sets are re-assembled for each experiment to show the robustness and repeatability. The result, tabulated in Table II, shows that the performance of our method was slightly degraded from 0.246N in MAE and 0.416N in RMSE to 0.287N in MAE and 0.462N in RMSE. However, the other two model show severe performance degradation. Especially, the MAE and RMSE of LSTM+BoA became 0.719N and 1.208N, respectively, which are almost twice as high as the initial results. This proved that our model outperforms existing deep learning models in repeatability. We also included the Standard Deviation of absolute error in the table, which was 0.362N. The manuscript has been modified accordingly as follows. Action: The modifications have been made in the manuscript as follows: - Table II (Results of Fingertip Force Estimation for Repeatability) has been inserted. - A new section has been added in Result and Analysis. F. Further Analysis 5: Repeatability - IV. Results and Analysis, F. Further Analysis 5: Repeatability (Paragraph 1) -Added “In order to evaluate the repeatability, additional experiments were performed to gather three different test datasets. Each of the test datasets was collected 1) in accordance with the aforementioned data collection method, 2) on a different day, and 3) by re-assembling the actuation system. The results are tabulated in Table. II. The performance of the proposed method was slightly decreased from 0.246N in MAE and 0.416N in RMSE to 0.287N in MAE and 0.462N in RMSE. However, the other two methods show severe performance degradation. The performance of LSTM+BoA was decreased from 0.361N in MAE and 0.596N in RMSE to 0.719N in MAE and 0.971N in RMSE. Standard deviations of the absolute errors for each method were also displayed. BT-LSTM had the standard deviation of 0.362N while LSTM+BoA and LSTM had the standard deviations of 0.971N and 1.517N, respectively.” - V. Discussion and Conclusion (Paragraph 1) - Modified “… Then, repeatability test was performed using three different datasets collected on different days.”

Minor comments:

In Figure 2, 5 and 6: the authors should use different line styles for the graph for simplified readability.

Response: We thank the reviewer for the feedbacks. All the changes proposed by the reviewer have been implemented in the manuscript. Action: We implemented all changes suggested by the reviewer.

Reviewer: 6

Comments to the Authors:

This paper proposes a method to estimate the generated force at the fingertip of a wearable hand robot using the tendon-sheath mechanism. In order to deal with the non-linearity derived from the bend of the sheath and compliance of the soft/flexible structure, which are the problems in the estimation, the authors integrate a sensor that measures the bend of the sheath and a deep learning method that focuses on time-gradient changes. The results of experiments and analysis show that the proposed method has higher estimation accuracy than the existing methods.

But, there are some unclear points about the essential idea of the method and its implementation.

Major concerns:

1) In the abstract, the essential idea of the proposed method should be explained, and the evaluation results should be presented quantitatively.

Response: We greatly thank the reviewer for the valuable comments. The essential idea of the proposed method is to estimate fingertip forces in real-time without sensors on the glove. This paper also tries to verify the robustness of the proposed method by showing the performance under dynamic environments. BT-LSTM, the proposed deep learning algorithm, integrates dynamic information of motor encoders, wire tension, and sheath bending angles. To clarify the essential idea of the proposed method, we have modified the abstract. Furthermore, we modified the abstract to better address the issue we are trying to solve, which will help to understand the essential idea of the proposed method. Also, the abstract was modified to include quantitative evaluation results. After submission, we have conducted two further experiments to analyze performance of the proposed method. The first experiment was conducted to verify the repeatability of the method. Three different test datasets, which were collected on a different day after re-assembling the actuation system. Even with the new datasets, the proposed method showed best performance compared to the other methods. The second experiment was conducted to show the real-time applicability and force control. A PID controller was implemented and tested using 5-second step function. The detailed results of the experiments were included in the manuscript. Based on these results, the abstract was modified to present quantitative results. Action: The modifications made in the new manuscript are listed below: - Abstract - Modified “Soft wearable hand robots with tendon-sheath mechanisms are being actively developed to assist people with lost hand mobility. For these robots, accurately estimating fingertip forces leads to successful object grasping. An approach can utilize information from actuators assuming quasi-static environments. However, non-linearity and hysteresis with regards to the dynamic changes of the tendon-sheath mechanism hinder accurate fingertip force estimation. This paper proposes a learning-based method to estimate fingertip forces by integrating dynamic information of motor encoders, wire tension, and sheath bending angles. The model is modified from Long Short-Term Memory by incorporating a residual term that governs the dynamic changes in sheath bending angles. Using a tendon-driven soft wearable hand robot, the proposed model obtained RMSE less than 0.44N. It was further evaluated under criteria ranging from different object sizes, bending angle ranges, and forces. Finally, a repeatability test (0.46N in RMSE), real-time applicability (125 Hz), and force control (12.7% in MAPE) were performed to verify the feasibility of the proposed method.” - IV. Results and Analysis, F. Further Analysis 5: Repeatibility (Paragraph 1) -Added “In order to evaluate the repeatability, additional experiments were performed to gather three different test datasets. Each of the test datasets was collected 1) in accordance with the aforementioned data collection method, 2) on a different day, and 3) by re-assembling the actuation system. The results are tabulated in Table. II. The performance of the proposed method was slightly decreased from 0.246N in MAE and 0.416N in RMSE to 0.287N in MAE and 0.462N in RMSE. However, the other two methods show severe performance degradation. The performance of LSTM+BoA was decreased from 0.361N in MAE and 0.596N in RMSE to 0.719N in MAE and 0.971N in RMSE. Standard deviations of the absolute errors for each method were also displayed. BT-LSTM had the standard deviation of 0.362N while LSTM+BoA and LSTM had the standard deviations of 0.971N and 1.517N, respectively.” - IV. Results and Analysis, G. Further Analysis 6: Real-time applicability and Force Control (Paragraph 1) -Added “This section is to examine the feasibility for an actual deployment of the proposed system. Two computing devices were tested for real-time applicability: a laptop (ROX Zephyrus GX501, Asus) and an embedded computing device (Jetson AGX Xavier, NVIDIA). On the laptop, the fingertip forces were estimated with 125Hz, and the embedded device gave 22.2Hz.”

2) In the introduction, although it can be understood that the proposed method, BT-LSTM, can handle nonlinearity of sensor information using time-gradient, the essential idea underlying the technique is unclear. In other words, it is desirable that the applicability of the proposed method to other problems is shown.

Response: We thank the reviewer for the insightful comment. The essential idea of BT-LSTM is to consider nonlinear behavior terms, such as sheath bending angle, using BT term. BT-LSTM is an algorithm that can be implemented to any other robotic systems that uses remote actuator system with TSM, such as other wearable robots or medical robots. It is notable that some of the references in the introduction are from medical robot applications, which were also suffering from nonlinear behaviors of TSM. Furthermore, it is important that BT term is not limited to sheath bending angles. Any other nonlinear behaviors can be implemented into BT term. To be clearer, we also modified the algorithm section in the manuscript, which will be further explained in the next comment response. Thanks to the reviewer’s comment, we have modified our introduction to include our essential idea underlying the technique. Furthermore, in the discussion section, the manuscript has been modified to show the applicability of the proposed method. Action: The modifications have been made in the manuscript as follows: - I. Introduction (Paragraph 6) “…As a learning model, Bending Time-gradient Long-Short Term Memory (BT-LSTM), a modified version of Long-Short Term Memory (LSTM), is proposed to deal with obtained non-linear information obtained from the TSM. BT-LSTM has a residual structure that inputs bending-time gradient, which incorporates with the hidden outputs of LSTM. Adding this structure enables to better deal with the dynamic changes of the angle displacement. Performances of BT-LSTM were compared with conventional algorithms to estimate fingertip forces for various object sizes and sheath bending angles for verification. Considering sheaths movements during grasping motions, the performance of the model was evaluated by estimating fingertip forces under the dynamic change of sheath bending angles. In addition, a repeatability test, real-time applicability, and force control experiment were performed to verify the feasibility of the proposed method in real-life application.” - The section title “Conclusions” has been modified to “Discussion and Conclusion”. - V. Discussion and Conclusion (Paragraph 4) - Modified “Furthermore, The proposed method can be used to manage other non-linear behaviors by using BT term of the algorithm. Thus, in the future, the proposed method has potential to be implemented into any robotic systems that uses remote actuation systems with TSM, such as medical robots.”

3) The specific content of x_ {bf,t} in equation (6) is unclear. How is this data created from sensor information?

Response: We thank the reviewer pointing out the unclear expressions about the BT term calculation that can strengthen our paper. We modified the expression as follows: BT=∇xt(7)∙tiTρt As seen, the BT term is calculated by the gradient of the bending angle at the flexion wire given time multiplied by tiT, to the power ρt. Since the input data are discrete, the gradient can simply be calculated by the difference between the bending angle value at t and the value at t-1, which can be expressed as ∆xt(7)=Δxt(7)-Δxt-1(7). ρt is a trainable power term; we train this term as well as the other LSTM weights. After calculations, T-1 number of BT terms is obtained for each operation, from t=2 to T. Each of BT Term is multiplied by the hidden output at t, ht, and memory output at time t, ct. In order to minimize the misunderstanding of the notations, we modified terms mentioned in <III. Method – C. Fingertip Force Learning 1) Input and Output Data> to accord the notations of LSTM: time step t becomes lowercase and the input numbers become uppercase with parentheses. Action: The modifications have been made in the manuscript as follows: - III. Methods – C. Fingertip Force Learning 1) Input and Output Data (Paragraph 1) “In a LSTM window with the assigned time window value T, an input data xt at each time stept, where t= 1,2,...,T, is composed as follows: xt={xt1, xt2, xt3,xt4,xt5,xt6,xt7,xt8} … and yT is the fingertip force at time T. A sequence of the sensor output x(1:T)={xt=1,xt=2,…,x(t=T)}” - III. Methods – C. Fingertip Force Learning 2) BT-LSTM The equation (6) has been modified as follows. BT=∇xt(7)∙tiTρt - III. Methods – C. Fingertip Force Learning 2) BT-LSTM (Paragraph 2) “… Unlike the usual LSTM architecture, the BT term in BT-LSTM calculates the gradient of bending angle at the flexion wire given time, ∇xt(7), which can be obtained by Δxt(7)-Δxt-1(7). This term is then multiplied by tiT where ti represents the current time index in the time window.”

4) Regarding the data in Fig. 7, it is difficult to confirm the correlation between the time when the estimation error occurs and the time when the bending angle change occurs. The data of LSTM+BoA has spikes, but it also has spikes in the data in Fig 5. For this reason, it is difficult to understand that the error is caused by a dynamic change in the bending angle of the sheath.

Response: We thank the reviewer for the valuable comment, and we agree that the error source of Fig. 7 can be unclear. The difference between Fig. 5 and Fig. 7 is that for Fig. 7, we intentionally wired or unwired the tendon-sheath to collect data during grasping and releasing while we did not for Fig 5. However, the bending angles constantly change during grasping even if we do not intentionally manipulate the tendon-sheath. Therefore, technically, it is not easy to know that the spike comes from the intentional rapid bending angle changes or other sources. The main objective for displaying Fig. 7 is that the dynamic angle changes can affect the performance more on the existing LSTM-based models than ours (the dynamic bending angle changes can happen a lot when a user move the user’s arm toward an object). Still, we conducted two additional works to explain this as best as we could. First, we drew vertical lines where the bending angles were sharply changed in Fig. 7. By doing this, it is possible to correlate the bending angles and predicted fingertip forces. Second, we performed additional analysis to compare the RMSE of the data having the largest bending angle changes and the smallest bending angles changes. The cumulative changes in bending angles were calculated as follows: Cumulative changes in bending angle=t=2Txt(7)-xt-1(7) Where xt(7) and xt-1(7) are the data of sheath bending angle on the flexion wire at time t and t-1, respectively. We sorted the changes in bending angle data, and calculated RMSE. The top 5000 data for the smallest bending angle changes and the largest bending angle changes were compared. Our model for the smallest bending angle changes was 0.26N while the data for the largest angle changes was 0.44N. LSTM+BoA gave 0.32N (smallest) and 0.75N (largest). LSTM did not show accurate performance on both, having 0.99N (smallest) and 1.1N (largest). Based on this result, we can infer that the dynamic changes in bending angle affect the performances of fingertip force estimation while our model had the less impact of the bending angle changes than LSTM+BoA. Action: The modifications have been made in the manuscript as follows: - IV. Results and Analysis, E. Further Analysis 4: Bending Angle Changes During Grasping (Paragraph 2) -Added “To quantitatively analyze the impact of dynamic angle changes, the top 5000 data that had the largest cumulative bending angle changes and the smallest cumulative bending angle changes were compared in RMSE. The cumulative changes in bending angle were calculated by t=2Txt(7)-xt-1(7). The proposed model had 0.26N (top 5000 smallest) and 0.44N (top 5000 largest). LSTM+BoA had 0.32N(smallest) and 0.75N (largest). LSTM did not seem to produce accurate results for both smallest and largest changes in bending angles, which had 0.99N (smallest) and 1.1N (largest). Based on the result, the proposed method had the less impact of the bending angle changes compared to LSTM+BoA.” - Fig. 7 has been modified.

Minor comments:

1) The proposed method seems to assume a state in which contact force is generated only at the fingertips. If so, it would be less misleading to mention it in the introduction.

Response: We thank the reviewer for the feedback. We have modified the manuscript as the reviewer suggested to prevent readers from the misleading. Action: To clarify, the modifications made in the manuscript are as follows. - I. Introduction (Paragraph 6) “Assuming the contact force is generated only at the fingertips, the proposed approach can estimate fingertip forces regardless of variation of object sizes and dynamic change in sheath bending angles.”

2) Figure 2 shows experimental data in which flexion and extension are continuously performed. It would be helpful to show which part in the graph is data of flexion process and which part is data of extension process.

Response: We thank the reviewer for the feedbacks. The change proposed by the reviewer has been implemented in the manuscript. Action: Fig. 2 was modified as the reviewer suggested.

3) There are two unclear points regarding the additional analysis of the difference in object size. First, how many millimeters is the exact width of the objects 1 to 5? Second, why is the object 3 less accurate and the object 5 more accurate?

Response: We thank the reviewer for the comment. The exact width of the object was 30mm for all objects. As mentioned in the minor comment 1), throughout the experiment, the width of the object was small enough only for the fingertip to contact the objects. If you are referring “width” as the distance between thumb and index finger (In the manuscript, we referred it as the size of the dynamometer), the numbers are mentioned in <III.Methods-A.Hardware setup-1)Overall system>. Object 1 was 68mm and object 5 was 88mm. Each object had 5mm size increment. To clarify the point, we have modified the manuscript. For the second question, the reason for the object 3 being less accurate is unclear. However, we presume that the object 3 could be a slightly gray area for the deep learning algorithm compared to object 1 and 5, since the object size difference is small. We believe that the issue will be mitigated by increasing the datasets. Thanks to the reviewer, we have added explanation on the result in the manuscript. Action: The modifications made in the manuscript are as follows. - III. Methods – A. Hardware Setup – 1) Overall System (Paragraph 2) “Thus, the sizes of the dynamometer, the distance between the thumb and index finger, are 68mm, 73mm, 78mm, 83mm, and 88mm, for object 1 to 5, respectively.” - IV. Results and Analysis – B. Further Analysis 1: Different Object Sizes “It is presumed that object 3 could have a lot of confused data points from the neighboring object sizes, compared to other objects because the differences in object size are small.”

4) There is a typo "direct placing placing sensors on ..." on the 6th line of the abstract.

Response: We thank the reviewer for the feedbacks. All the changes proposed by the reviewer have been implemented in the manuscript. Action: We implemented all changes suggested by the reviewer.

5) Units should be written in roman rather than italic.

Response: We thank the reviewer for the feedbacks. All the changes proposed by the reviewer have been implemented in the manuscript. Action: We implemented all changes suggested by the reviewer.