Hello. I am gayoung Yang from intelligence robotics lab from Sejong Univ.

Today, I'm here to discuss our research about Vision-Based Intelligent Gait Environment Detection Algorithm for Assisting Stroke Patients with a Soft Ankle Exosuit.

------------------------------------------------------------------------------------------------

Here is the content of this presentation

------------------------------------------------------------------------------------------------

Stroke patients often experience functional impairments with gait disturbances.

One common symptom is foot drop, which leads to reduced foot clearance during the swing phase of gait cycle.

In daily life, individuals frequently encounter not only level ground but also stair environments, which pose greater challenge due to increased demands on the joint range and muscular moments in the lower extremities.

Particularly, during stair ascent, reduced foot clearance due to foot drop can results in insufficient clearance between the foot and the stair nosing.

Consequently, a lower limb exoskeleton has been developed to assist with gait in stair environments.

------------------------------------------------------------------------------------------------

Prior research has utilized insole and IMU sensors to monitor the current state of the gait.

However, these sensors lack prior information about the environment, limiting the capacity to anticipate environmental changes in advance.

To address this issue, vision-based environmental perception algorithms for exoskeletons were conducted.

This approach mimics the human vision system, enhancing awareness of the surrounding environment and provides guidance for locomotion.

------------------------------------------------------------------------------------------------

Although vision-integrated exoskeletons have been developed, improving performance during environment transitions remains challenging and exoskeletons need to be developed with considerations for stroke patients.

To enhance the performance, deep learning-based methodologies, with recent advancements influencing robotics, can be employed.

In this study, we developed a vision-based intelligent gait environment detection algorithm for ankle exosuit. This approach integrates deep learning-based environment classification into the exoskeleton application, utilizing YOLO for detailed gait environment classification and LSTM for spatio-temporal feature extraction.

------------------------------------------------------------------------------------------------

The soft ankle exosuit is constructed from flexible and lightweight materials that conform to movements, providing assistive forces through a tendon - driven mechanism.

The exosuit consists of vest, waist belt, leg brace, and foot wearable components, with a chest- mounted camera to capture the gait environment.

IMU is attached to the shin and thigh positions of the leg brace, respectively.

Toe-off detection utilizes shank angular velocity and thigh angular velocity from IMU sensors.

For level walking, dorsiflexion is assisted at toe-off, and plantarflexion is supported at foot-flat phase for horizontal foot alignment.

During stair ascent, increased force assists dorsiflexion at toe-off phase, and during descent, dorsiflexion assistance at toe-off is followed by plantarflexion before heel strike, ensuring stability.

------------------------------------------------------------------------------------------------

A customized dataset was constructed by recording a single female participant walking on both level ground and stair environments.

Each frame in videos was captured at a resolution of 960 × 540 (nine hundred sixty by five hundred forthy) pixels.

The dataset was partitioned into 8:1:1(eight to one to one) ratio with augmented train images.

It was categorized into seven distinct gait environments: level ground, initial incline, incline, final incline, initial decline, decline, and final decline.

------------------------------------------------------------------------------------------------

The YOLO algorithm is an object detection framework that adopts a one-stage detection approach, creating a simplified pipeline for rapid real-time detection.

Additionally, by processing input images globally, it effectively uses contextual information and achieves strong generalization.

In our study, rapid response to gait environment transitions and spatial analysis is crucial, making the YOLO model an ideal choice due to these capabilities.

The YOLO model is trained with optimal parameters within the following framework, and outputs the probability of seven classes and identifies the top-1 class for each detection.

However, gait is a form of time-series data in which characteristics such as the gait environment and joint angles change over the time domain.

To accurately classify the gait environment, it is essential to consider both past and future contexts relative to the current frame.

Accordingly, an LSTM model, which was trained on sequential time-series data, was selected.

------------------------------------------------------------------------------------------------

By providing detailed context about the current gait environment through the YOLO output, the LSTM extracted relevant features for each phase of stair negotiation.

The videos were extracted at 20 frames per second, labeling one class for every ten frames. A stride of five was used, resulting in four labels per second. The LSTM data shape was (1714, 10, 8), consisting of sequences with ten time steps and eight features.

For the LSTM data preparation, a maximum voting technique was applied to label the most probable class.

In cases of a tie, an average voting technique was used, selecting the class with the highest average probability.

The LSTM model was trained for 1000 epochs with a batch size of 32 and a learning rate of 0.001, and it utilized the Adam optimizer.

------------------------------------------------------------------------------------------------

To evaluate the real-time performance of the gait environment classification algorithm, we conducted an experimental study with a female participant.

The participant walked at a constant speed, ascended an eight-step staircase, followed a circular path on level ground, and then descended the staircase to return to the start.

To assess the performance of the ankle exosuit, the experiment was conducted under three conditions: (1) without exosuit (no exo), (2) exosuit worn but unpowered (exo off ), and (3) exosuit worn and powered (exo on).

The participant underwent a 10-minute familiarization with the exosuit and a 20-minute rest interval was implemented between each experimental condition to prevent residual effects from subsequent conditions.

The inertial motion capture system was attached to participant’s foot and mid-shank to analyze the ankle joint angles in the sagittal plane.

Surface electrodes were placed on the tibialis anterior and medial gastrocnemius muscles to measure activation, with EMG data normalized to pre-measured maximum voluntary contraction values.

------------------------------------------------------------------------------------------------

The YOLO system showed low recall in transition classes, particularly for IC prev and DC prev, resulting in an F1 score of 0.774 due to misclassifications. In contrast, the YOLO-LSTM integration achieved an F1 score of 0.971, indicating balanced performance.

This combined system improved precision, recall, F1 score, and accuracy, enhancing real-time transition timing.

The YOLO model misclassified level ground as stairs 1.5 seconds before the actual transition, while YOLO-LSTM enabled rapid and accurate classification.

The system detected environmental changes with a timing variation of 0.25 seconds, which did not impact the exosuit's performance.

When misclassifications occurred in the YOLO model, the LSTM compensated by analyzing previous context and sequence patterns to make accurate predictions.

This highlights the importance of spatio-temporal feature extraction in adaptive exoskeletons, enabling rapid and precise responses to dynamic environmental changes and improving system adaptability and real-time performance.

------------------------------------------------------------------------------------------------

The average ankle joint angles for each experimental condition are shown in the figure.

The analysis revealed a significant increase in dorsiflexion during the "exo on" condition on level ground.

This increase can help alleviate foot drop symptoms in stroke patients and improve foot clearance during gait.

In stair environments, notable improvements in ankle angles were observed in both the "exo off" and "exo on" conditions.

------------------------------------------------------------------------------------------------

During the stair ascent and level ground, the exosuit reduced the tibialis anterior muscle activations, compared with the unworn condition. In contrast, during the stair descent, the exosuit demonstrated similar muscle activation regardless of its powered state. Gravitational forces decrease the load on the ankle joint during a stair descent, which may naturally reduce muscle activation without the exosuit.

------------------------------------------------------------------------------------------------

Additionally, the exosuit reduced the MG muscle activations across overall condition. This indicates effective support for ankle movements during stair ascent, reducing muscle activation while maintaining similar ankle angles.

------------------------------------------------------------------------------------------------

This research presents a vision-based gait environment detection algorithm for an ankle exosuit designed for stroke patients. By combining YOLO's object classification with LSTM for spatio-temporal pattern recognition, the system enhances environmental recognition and response times. The analysis shows that the exosuit significantly increases ankle dorsiflexion during level walking, helping to alleviate foot drop symptoms and improve foot clearance. During stair ascent, the exosuit reduces TA and MG muscle activation, demonstrating effective support for ankle movements while maintaining similar angles.

Future work should focus on validating these results in diverse real-world scenarios and with multiple subjects, particularly stroke patients, to further establish the exosuit’s effectiveness and generalization ability.

------------------------------------------------------------------------------------------------This paper is published in the Actuators Special Issue: Recent Advances in Soft Actuators, Robotics, and Intelligence. This issue highlights the latest developments in soft technologies for robotics. If you're interested, we encourage you to submit your high-quality manuscripts addressing key challenges in this field.

Thank you for your attention, and I welcome any questions or discussions on our research.